# IoTbase RPi 3-04 Test Summary

Test Date: February 2026

## EEPROM Tests

Full test and report generation:

python3 iotbase_eeprom_report.py --chip 24c08 --base 0x54 --bus 1 --verbose

Artifacts: reports/20260204_103504_report.txt, reports/20260204_103504_results.json

Retention-only verification after power cycle:

python3 iotbase_eeprom_report.py --chip 24c08 --base 0x54 --bus 1 --verify reports/<timestamp>_midblock.bin

Artifacts: reports/20260204_104048_report.txt, reports/20260204_104048_results.json

## WatchDog Tests

Non-destructive testing:

python3 iotbase_watchdog_report.py --mode feed-only --timeout 5 --run-seconds 180 --period 0.5

Artifacts: reports/20260204_104501_wdt_report.txt, reports/20260204_104501_wdt_report.json

Real test with reboot:

python3 iotbase_watchdog_report.py --mode trigger-reboot --timeout 5 --run-seconds 10 --period 0.5

Artifacts: reports/20260204_104606_wdt_pretrigger.txt, reports/20260204_104606_wdt_pretrigger.json

Post-check after restart:

python3 iotbase_watchdog_report.py --mode post-check

Artifacts: reports/20260204_104859_wdt_postcheck.txt, reports/20260204_104859_wdt_postcheck.json

## GPIO Pair Tests

| Test Pair | Command | Result | Status |
| --- | --- | --- | --- |
| AP2 ↔ AP3 | iotbase_gpio_pair_report.py AP2 AP3 --bidir | pass=8 fail=0 | PASS |
| AP0 ↔ AP1 | iotbase_gpio_pair_report.py AP0 AP1 --bidir | pass=8 fail=0 | PASS |
| AP4 ↔ AP5 | iotbase_gpio_pair_report.py AP4 AP5 --bidir | pass=8 fail=0 | PASS |
| AP6 ↔ AP7 | iotbase_gpio_pair_report.py AP6 AP7 --bidir | pass=8 fail=0 | PASS |

## HOST-P12 I/O Test

python3 hostp12_io.py

Artifacts: reports/20260204_110512_hostp12_inputs.txt, reports/20260204_110512_hostp12_inputs.json

## RS485 Communication Test

python3 iotbase_rs485_echo_test.py

Artifacts: reports/20260209_163359_rs485_echo_test.txt, reports/20260209_163359_rs485_echo_test.json

Note: Requires echo server running on PC (echo_server.py)

Configuration: Jumpers in AUTO position, /dev/ttyAMA0, 115200 baud

## Test Summary

| Component | Status | Notes |
| --- | --- | --- |
| EEPROM (24C08) | PASS | All tests passed |
| WatchDog | PASS | Reboot triggered OK |
| GPIO Pairs (AP0-AP7) | PASS | 32/32 tests passed |
| HOST-P12 I/O | PASS | Snapshot OK |
| RS485 Communication | PASS | TX/RX verified |

ALL TESTS PASSED - BOARD FUNCTIONAL